## How to Build and Deploy Code for nRF52840 Dongle

### Step 1: Clone the Project

- Open Command Prompt (CMD).

- Navigate to the folder where you want to save the project.

cd C:\Users\TXR\Desktop\flash_pod

3. Clone the GitHub repository.

git clone https://github.com/txr-academy/FlashPod

After cloning, the project structure will look like:

FlashPod/

├── esp32code/

├── nrf52840code/

├── README.md

### Step 2: Prepare the nRF52840 Project

- Open Visual Studio Code.

- Go to File → Open Folder.

- Open the nrf52840code folder.

#### Create the Required Folder Structure

Create a folder named boards and move the overlay file into it.

boards/

└── nrf52840dongle_nrf52840.overlay

Create another folder named src and move all source and header files into it.

src/

├── main.c

├── rgb_led.c

├── rgb_led.h

├── buzzer.c

├── buzzer.h

├── ir_sensor.c

└── ir_sensor.h

The final project structure should be:

nrf52840code/

├── boards/

│   └── nrf52840dongle_nrf52840.overlay

├── src/

│   ├── main.c

│   ├── rgb_led.c

│   ├── rgb_led.h

│   ├── buzzer.c

│   ├── buzzer.h

│   ├── ir_sensor.c

│   └── ir_sensor.h

├── CMakeLists.txt

└── prj.conf

### Step 3: Build the Project in VS Code

- Open the nRF Connect extension in VS Code.

- Click Applications.

- Select Add Build Configuration.

- Configure the build with the following settings:

  - Board Target: nrf52840dongle/nrf52840

  - Build Directory: Default

  - Configuration: Default (prj.conf)

- Click Generate and Build.

After a successful build, the output files will be generated in the build directory, including:

build/

└── zephyr/

├── zephyr.hex

└── merged.hex

The merged.hex file is used for programming the nRF52840 Dongle.

### Step 4: Flash the Firmware

- Connect the nRF52840 Dongle to your PC.

- Open nRF Connect for Desktop.

- Launch the Programmer application.

- Select the connected nRF52840 Dongle.

- Click Add File and browse to:

- build\zephyr\merged.hex

- Click Write (or Erase & Write) to program the firmware.

Once flashing is complete, the firmware will start running on the nRF52840 Dongle.